ISO TC 188/SC  N

Date:   2004-05-18

ISO/CD 12215-8

ISO TC 188/SC /WG 18

Secretariat:   SIS

Hull construction - Scantlings — Rudders

Construction de la coque -Echantillonnage — Gouvernails

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Originator:  ISO/TC 188/WG 18 Scantlings
Convenor:  Mr. Grégoire DOLTO e-mail: dolto@fin.fr

For the draft reader:

The ISO time schedule pushes WG 18 of TC 188 to produce a CD, whereas the contents of the drafts are still preliminary and open to discussion. This CD is therefore mainly set of articles listing the items to be considered and proposing methods of analysis, rather than a comprehensive draft. The details are, in many cases, still to be implemented or developed, this will be done for the DIS.

The aim of this CD is to have the international ISO community to validate the general structure of the draft rather than approved a finished draft.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	vi

Introduction	vii

1	Scope	1

2	Normative reference	1

3	Terms and definitions	1

4	Symbols	2

5	Rudder and steering arrangement, rudder types	3

5.1	General	3

5.1.1	General definition	3

5.1.2	Multi-rudder arrangement	4

5.1.3	Vertical support	4

5.1.4	Stops	4

5.1.5	Actuating system of the rudder	4

5.1.6	Emergency tiller.	4

5.2	Rudder types	4

5.2.1	Spade rudder : Type I (See Figure 1)	4

5.2.2	Other rudder types (See Figure 2)	5

6	Speed and side force calculation	7

6.1	Speed for calculation	7

6.1.1	Speed of motor craft	7

6.1.2	Speed of sailing craft	7

6.1.3	Maximum rudder force coefficient CR	8

6.1.4	Side force on the rudder	8

7	Rudder stock bending moment and torque	9

7.1.1	Design rudder stock bending moment M	9

7.1.2	Design rudder stock torque T	9

8	Rudder stock design stress	10

8.1	Metal rudder stock material	10

8.2	Design stress for metallic rudder stock	10

8.3	Metallic Stock Diameter	11

8.3.1	Solid circular stock diameter	11

8.3.2	Tubular stocks	12

8.3.3	Non-circular metal rudder stocks	12

8.4	Simple non isotropic rudder stocks (i.e wood)	13

8.5	Complex composite rudders	13

9	Equivalent diameter in way of notches	13

10	Rudder bearings, pintles and gudgeons	13

10.1	Bearing arrangement	13

10.1.1	Type I (spade)	14

10.1.2	Types II, III and IV (rigid skeg)	14

10.1.3	Type V (rigid skeg)	14

11	Rudder stock structure and rudder construction	14

11.1	Rudder stock structure	14

11.2	Rudder construction	15

11.3	FRP rudder blades	15

11.3.1	Blades with a core	15

11.3.2	Blades skins without internal core	15

11.4	Non-FRP rudder blades	16

Annex A (normative)  Metal for rudder stock	17

A.1	Metal rudder stock material	17

A.1.1	General	17

A.1.2	Stainless steel.	17

A.1.3	Aluminium alloys.	17

A.1.4	Titanium alloys	17

A.2	Design stress for metallic rudder stock	17

Annex B (informative)  Complex composite rudder stock design	19

B.1	Scope	19

B.2	Use of design loads	19

B.3	Design Criteria	19

B.3.1	Strength criteria:	19

B.3.2	Stiffness criteria:	20

B.4	Methods of analysis	20

B.5	Other considerations	20

B.5.1	Skin wrinkling	20

B.5.2	Semi-spade rudders.	20

Annex C (informative)  Technical background	25

C.1	Background documents	25

C.1.1	Sailing boats	25

C.1.2	Motor boats	25

C.2	Hypothesis of calculation	25

C.2.1	General	25

C.2.2	Sailing boats	25

C.2.3	Motor boats	26

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies

International Standard ISO 12215 was prepared by Technical Committee ISO/TC 188, Small craft.

Beside this eight part, ISO 12215 consists of

- Part 1: Materials - Thermosetting resins, glass fibre reinforcement, reference laminate

- Part 2: Materials - Core materials for sandwich construction, embedded materials

- Part 3: Materials - Steel, aluminium, wood, other materials

- Part 4: Workshop and manufacturing

- Part 5: Design pressures for monohull, design stress, scantlings determination

- Part 6: Structural arrangements and details

- Part 7: Multihulls

- Part 8: Rudders

- Part 9: Appendages and rig attachments

The development of ISO 12215 parts 1 to 9 owes a considerable debt to the energy and work of Mr Fritz HARTZ who was involved at the start of the project and was the convener of TC 188 WG 18 until his death on the 16th of November 2002. All the members of WG 18 and TC 188 wish to express their gratitude for his major contribution to the production of this international standard

Introduction

The reason underlying the preparation of this International Standard is that standards and recommended practices for loads on the hull and the dimensioning of small craft differ considerably, thus limiting the general world wide acceptability of boats.

The objective of this standard is to achieve an overall structural strength that ensures the watertight and weathertight integrity of the craft.

The working group considers this standard to have been developed applying present practice and sound engineering principles. The design pressures of this standard shall be used only with the equations of this standard.

Considering future development in technology and boat types, and small craft presently outside the scope of this standard, provided methods supported by appropriate technology exist, consideration may be given to their use provided equivalent strength to this standard is achieved.

The dimensioning according to this standard is regarded as reflecting current practice, provided the craft is correctly handled in the sense of good seamanship and equipped and operated at a speed appropriate to the prevailing sea state.

Hull construction - Scantlings — Rudders

# Scope

This part of ISO 12215 gives requirement on the scantlings of rudders fitted to small craft with a length of hull (LH) of up to 24 m, measured according to ISO 8666. It applies both to monohulls and multihulls.

It does not give requirements on rudder characteristics required for proper steering capabilities.

NOTE	Scantlings derived from this standard are primarily intended to apply to recreational craft including charter boats.

# Normative reference

The following normative documents contain provisions which, through reference in this text, constitute provisions of this International Standard. For dated references, subsequent amendments to, or revisions of, this publication do not apply. However, parties to agreements based on this International Standard are encouraged to investigate the possibility of applying the most recent edition of the normative document indicated below. For undated references, the latest edition of the normative document referred to applies. Members of IEC and ISO maintain registers of currently valid International Standards.

ISO 8666: — Small craft — Principal data

ISO 4948-1 —,Steels  — Classification —  Part 1: Classification of steels into unalloyed and alloy steels based on chemical composition

ISO 4948-2 —,Steels  — Classification —  Part 2: Classification of steels into unalloyed and alloy steels according to main quality classes and main property of application characteristics

# Terms and definitions

For the purposes of this International Standard, the following terms and definitions apply.

3.1
design categories

sea and wind conditions for which a boat is assessed by this International Standard to be suitable, provided the craft is correctly handled in the sense of good seamanship and operated at a speed appropriate to the prevailing sea state.

3.1.1
design category A ("ocean")

category of boats considered suitable to operate in seas with significant wave heights above 4 m and wind speeds in excess of Beaufort Force 8, but excluding abnormal conditions, e.g. hurricanes.

3.1.2
design category B ("offshore")

category of boats considered suitable to operate in seas with significant wave heights up to 4 m and winds of Beaufort Force 8 or less

3.1.3
design category C ("inshore")

category of boats considered suitable to operate in seas with significant wave heights up to 2 m and a typical steady wind force of Beaufort Force 6 or less

3.1.4
design category D ("sheltered waters")

category of boats considered suitable to operate in waters with significant wave heights up to and including 0,30 m with occasional waves of 0,5 m height, for example from passing vessels, and a typical steady wind force of Beaufort 4 or less

3.2
design category factor for rudder

fwR

factor lowering requirements according to design category, its values are according to Table 1

Table 1 — Values of design category factor for rudder

| Design Category | A &B | C & D |
| --- | --- | --- |
| Value of fwR | 1 | 0,75 |

3.3
loaded displacement mass m LDC

mass of the craft, including all appendages, when in the fully loaded ready for use condition as defined in ISO 8666.”

3.4
sailing craft

boat for which the primary means of propulsion is by wind power, having a total profile are As, defined in ISO 8666, expressed in m², of all sails that may be set at one time when sailing closed hauled (for the headsails it is the area of the fore triangle) of	 As > 0,07( m LDC)2/3

In the rest of this International Standard, non-sailing craft are called motor craft.

# Symbols

Unless specifically otherwise defined, the symbols shown in Table 2 are used in this part of ISO 1215.

Table 2 — Symbols, coefficients, parameters

| Symbol | Unit | Designation/Meaning of symbol | Reference/Article concerned |
| --- | --- | --- | --- |
| fWR | * | Design category factor for rudder | 3.2 |
| mLDC | kg | Loaded displacement mass | 3.3 |
| V | knots | Speed of calculation | 6.1.1 |
| VMIN | knots | Minimum calculated speed for motor boats | 6.1.1 |
| V | knots | Speed of calculation | 6.1.1 |
| LWL | m | Length at waterline, acc. to ISO 8666 in mLDCconditions | 6.1.1 |
| mLDCR | kg | Reference value for loaded displacement mass | 6.1.2 |
| KV | * | Light sailing boat speed coefficient | 6.1.2 |
| Λ | * | Geometric aspect ratio of the rudder | 6.1.3 |
| CR | * | Maximum side force coefficient | 6.1.3 |
| A | m² | Total area of the moving parts | 6.1.4 |
| F | N | Side force on rudder | 6.1.4 |
| M | Nm | Bending moment on the rudder stock | 7.1.1 |
| Kb | * | Rudder bending coefficient | 5.2.1, 7.1.1 |
| Zb | m | Effective bending moment lever ZB =Kb hr + hc | 5.2.1, 7.1.1 |
| hr | m | Average height of rudder (see Figure 1) | 5.2.1, 7.1.1 |
| hc | m | Vertical distance of centre of low bearing and rudder top | 5.2.1, 7.1.1 |
| T | Nm | Torque (Twisting moment)on the rudder stock | 7.1.2 |
| r | m | Horizontal distance rudder force to stock axis | 5.2.1, 7.1.2 |
| rmin | m | Minimum value of r | - 5.2.1, 7.1.2 |
| c | m | Rudder chord length at mid span | 5.2.1, 7.1.2 |
| u | m | Longitudinal distance leading edge to stock axis at mid span | 5.2.1, 7.1.2 |
| σ | N/mm² | Stress (ultimate, yield, design) | 8.2 |
| Zeq | m | Equivalent bending moment lever (bending Mt + Torque) | 8.3.1 |
| d316 | mm | Solid rudder stock diameter from AISI 316 Stainless steel | 8.3.1 |
| Km | * | Metal coefficient | 8.3.1 |
| do | mm | Outer diameter of tubular stock | 8.3.2 |
| di | mm | Inner diameter of tubular stock | 8.3.2 |
| R | N | Reaction force at stock bearing | 9.2 |
| hu | m | Vertical distance centre of lower bearing / rudder mid span | 5.2.1, 10.1 |
| ha | m | Vertical distance between centre of bearings | 5.2.1, 10.1 |
| t | m | Vertical distance centre of lower bearing / rudder centroid | 5.2.1, 10.1 |

# Rudder and steering arrangement, rudder types

## General

### General definition

The rudder and steering arrangement comprises all components necessary for manoeuvring the craft, from the rudder and the rudder operating gear to the steering position.

Rudder and steering equipment shall be so arranged to permit inspection.

It is a good practice that the rudder keeps the steering effect after grounding (for example, a spade rudder with the stock not going down to the bottom, so the rudder blade breaks without bending the stock)

### Multi-rudder arrangement

If the craft has several rudders, the following requirements apply to each one of the rudders.

### Vertical support

The rudder stock shall be held vertically and secured against axial movement.

### Stops

Rudder stocks that are actuated by remote control shall be fitted with stops.

### Actuating system of the rudder

The following devices shall be able to transmit the rudder torque T, as defined in 7.1.2, without exceeding their design strength, as defined in Article 7:

- The actuating device that turns the rudder including the tiller, rudder arm, quadrant;

- The connection between the rudder stock and the actuating device (cone, square, key);

- Stops that are provided at either end of the tiller, rudder arm or quadrant stroke.

The connection between the rudder stock and the actuating device shall be designed to ensure alignment between the rudder blade and the tiller, actuating arm, etc, and allow an ulterior instant checking of this alignment.

### Emergency tiller.

Any component of the emergency tiller, where fitted, shall be to transmit the rudder torque 0,5T, where T is defined in 7.1.2, without exceeding their design strength, as defined in Article 7.

## Rudder types

The standard is applicable to 7 types of rudder configuration: Type I to Type IV, as shown in Figure 1 & 2 . In all cases except case I c, the rudder blade is taken as rectangular or trapezoidal close to rectangular.

### Spade rudder : Type I (See Figure 1)

- A is the spade area																		(m²)

- hr 		is the average height of the rudder													(m)

- hc 		is the vertical distance between the centre of the lowest bearing and the top of the rudder at the stock location.																			(m)

- b1 and b2  are respectively the top and bottom chord, or its natural prolongation (see Type I c)		(m)

- Zb=Kb hr +hc	is the vertical distance between the middle of neck bearing and the rudder force, where Kbis given in Table 3																(m).

- r 	is the horizontal distance between the rudder force and the rudder's rotational axis, as defined in Table 4 and not to be taken less than rmin													(m)

- c is the chord length at rudder force level ( same as the centre of surface)						(m)

- u is, for rudder types I, II and IV, the horizontal distance between the leading edge of the rudder and the stock vertical axis at the height of centre of surface. For rudder types III and V, u is measured aft of the leading edge of the partial or full narrow skeg.  (See Figure 1 & 2)								(m)

Key

I a 	Typical fast motor boat spade rudder with low aspect ratio and cut out top aft to avoid ventilation.
I b 	Near rectangular shape
I c	Semi elliptical shape

Figure 1 — Spade rudders : Type I

NOTE	The marking with a shaded circle shows the geometric centre of surface. The force is located at the same height, but at a distance 0,3 c aft of the chord's leading edge

### Other rudder types (See Figure 2)

Same dimensions as for spade rudders, except

- A is the spade area, divided into A1 & A2 in Type V									(m²)

- A3 is the skeg area																(m²)

For Type V

h1 and h2	are the portions of hr above and below the skeg bearing 							(m)

ha	is the height between  of the hull and skeg bearing centres									(m)

t	is the height of the centre of area of the rotating part of the rudder below skeg bearing centre	(m)

NOTE	For simplification of calculation the position of the side force is considered to be at mid span of hr

Key

Type II		Supported by pintle and solepiece		Type III		Narrow full skeg
Type IV		Wide full skeg						Type V		Partial skeg

Figure 2 — Other rudder Types : Type II to V

# Speed and side force calculation

## Speed for calculation

### Speed of motor craft

- The speed of calculation V is the highest anticipated speed of the craft in knots

- V shall not be taken less than:   VMIN=2,4Lwl 												(knt)     (1)

### Speed of sailing craft

- V shall be taken as

V=2,7Kv LWL  for sailing craft                                                                                                      (knt)     (2)

where

      - KV=1 				for mLDC≥4,3 LWL3 and									    (3)

      - KV=2,65 LWL23mLDC2  		for mLDC<4,3 LWL3 	not to be taken greater than 2		    (4)

- mLDC	is the loaded displacement mass, as defined in 3.1							       (kg)

- LWL	 is the waterline length in loaded displacement m LDC								(m)

Figure 3 — Values of Kv according to LWL and m LDC

To use the figure, find the intersection of LWLand mLDCand find KV corresponding to this intersection, by interpolation if necessary.

### Maximum rudder force coefficient CR

The maximum force coefficient CR is a function of the aspect ratio  and boat speed V , it is given in Figure 3. For sailing craft and motor craft with a speed ≤ 10 knots this value corresponds to equation (5)

CR = 0,0275 Λ3 - 0,2926 Λ2+1,0583 Λ+0,2006 but shall not be taken greater than 1,5			     (5)

where

- Λ=hr2A	is the rudder geometric aspect ratio													     (6)

- hr 	is the average height of the rudder (see Figure 1 & 2)									(m)

- A	is the rudder Area (see Figure 1 & 2)												(m²)

This value shall be respectively multiplied by 0,9; 0,78; 0,72 and 0,66 for maximum speeds of 15; 20, 30 and  ≥ 40 knots but with a smaller range of aspect ratios. Figure 4 may be used for quick determination of CR.

Figure 4 — Variation of CR with aspect ratio and maximal speed

### Side force on the rudder

F=135 fWR CR V² ⋅A⋅ is the rudder design side force	    (N)     (7)

where

- fWR	is the design category factor for rudder defined in 3.2

- A	is the total surface area of the moving part(s) of the rudder without skeg (see Figure 1 &2), (m²)

- V	is the boat speed for calculation defined  in 6.1.1 or 6.1.2;								(knt)

- CR	is the rudder force coefficient defined in 6.1.3.

# Rudder stock bending moment and torque

### Design rudder stock bending moment M

M=F Zb is the design rudder stock bending moment                                                                      (Nm)      (8)

where

Zb=Kb hr +hc is the effective bending moment lever defined in 5.2.1                                      (m)         (9)

,where Kb is the rudder bending coefficient, given in Table 3.

Table 3 — Values of kb according to rudder type

| - Type | Skeg type | Kb |
| --- | --- | --- |
| Ia and Ib | None | - See a) |
| Ic | None | According to centre of surface b) |
| II | None | 0,16 |
| III & IV | Rigid c) | 0,16 |
|  | Flexible | According to centre of surface b) |
| V c) | Rigid c) | 0,16.h1.A1−0,5.h2.A2A1+A2 |
|  | Flexible | According to centre of surface b) |
| a) kb is the ratio between position of centre of surface divided by hr. For trapezoidal rudder shapes, it shall be chosen according the following table b2/b1; 1,00; 0,90; 0,80; 0,70; 0,60; 0,50; 0,40; 0,30; 0,20. kb; 0,50; 0,49; 0,48; 0,47; 0,46; 0,44; 0,43; 0,41; 0,39. Where b1 and b2 are respectively the top and bottom chords of the rudder blade |
| b) kb is the ratio between position of centre of surface divided by hr; It may be approximated as a rectangle and use a) |  |  |
| c) A skeg is considered rigid if (EI)skeg ≥ 2 (EI)rudder . In that respect rudder stock and eventual rudder blade shall be considered. This is usually not the case for metal rudder stock and GRP skeg. See Annex B |  |  |

### Design rudder stock torque T

T = F ⋅ r  is the design rudder stock Torque                                                                                        (Nm)    (10)

where

- F	 is the rudder force as defined in 4.3.3												(N)

- r ,and c  	are defined in 6.2, but r shall not be taken less than rmin								(m)

Table 4 — Values of r and r min according to rudder configuration

| - Type | r | rmin |
| --- | --- | --- |
| I | - 0,3 c - u | - 0,1 c |
| II | - 0,3 c - u | - 0,1 c |
| III | 0,5 c - u | 0,05 c |
| IV | 0,25 c - u | 0,05 c |
| V | 0,2h1hr+0,3.c | 0,1−0,05h1hr.c |

where

- u is, for rudder types I, II and IV, the horizontal distance between the leading edge of the rudder and the stock vertical axis at the height of centre of surface. For rudder types III and V, u is measured aft of the leading edge of the partial or full narrow skeg.  (See Figure 1 & 2)								(m)

# Rudder stock design stress

## Metal rudder stock material

Considerations on metal choice for rudder stock are given in Annex A

## Design stress for metallic rudder stock

Values of design stress from Table A 1 in Annex A shall be used for the metals shown unless derived from tests.

If derived from tests the mechanical properties σu and σy used in equation (11) shall be 90 % of the mean relevant tested value or the mean value minus two standard deviations whichever is the lesser.

For metal rudder stocks material not listed in Table A1, the design tensile load shall be the smallest of

σd=σu2 or 0,9 σy                                                                                                            (N/mm²)	(11)

where

- σdis the design tensile stress 													     (N/mm²)

- σu is the ultimate tensile strength 												     (N/mm²)

- σy is the yield tensile stress 													     (N/mm²)

## Metallic Stock Diameter

### Solid circular stock diameter

#### For stocks made of AISI 316L or 316 Stainless Steel:

zeq=zB2+0,75r2     																      (m)      (12)

d316=3,87F.zeq13																    (mm)     (13)

#### For other metals:

d=d316 Km																			    (mm)     (14)

where	Km=3176σd  is the metal coefficient, given in Table A1 for the most typical metals	    (mm)     (15)

#### Spade rudder (Type I): approximated variation of a solid rudder stock diameter

The bending moment M and Torque T vary with height, and so does the demand for the variation of stock diameter.

A simple manufacturing approximation is proposed in Figure 5:

- above the lower bearing the diameter is dmax until h1/ha =0,15 and it tapered above reducing to 0,5 dmax at the top.

- below the lower bearing the rudder stock is Tapered reducing at d/dmax= 0,5 for h2/hb =0,7

Caution: These values do not consider the eventual equivalent diameter reduction due to key notches or square sections as in Figure 5.

Figure 5 — Variation of d/d max or SM/SM n function of h1/ha or h2/hb

NOTE	The calculations are based on the assumption that T≤0,15 M

### Tubular stocks

Where round tubular rudder stocks are used, the outer and inner diameters shall comply with equation (16)

d=3do4−di4do                                                                                                                               (mm)    (16)

where

- d 	is the required diameter of solid round rudder stock as defined in  8.3.1.1			   (mm)

- do 	is the required outer diameter of round tubular rudder stock 						   (mm)

- di 	is the required inner diameter of round tubular rudder stock 						   (mm)

In order to prevent the risk of local buckling and to provide adequate local strength in way of bearings or any load inducing device (keys, tiller arms, etc), the wall thickness should not be less than 0,1 do.

Tabulated values are given in Table 5

Table 5 — Equivalent diameter de

### Non-circular metal rudder stocks

σd≥ σ²+3τ²                                                                                                                            (N/mm²)    (17)

where

- σ=MZb  is the bending stress   													(N/mm²)

- τ=TZt    is the shear stress   														(N/mm²)

- Zb           is the minimum section modulus under bending								(cm3)

- Zt           is the minimum section modulus under torsion								(cm3)

## Simple non isotropic rudder stocks (i.e wood)

For wood rudder stocks, equation (17) shall be fulfilled:

σσu2+ττu2<0,25                                                                                                                               (18)

where

- σu  is the ultimate flexural strength (modulus of rupture) parallel to the stock axis   	    (N/mm²)

- τu   is the minimum shear strength parallel to the stock axis						    (N/mm²)

## Complex composite rudders

Complex composite rudder stocks, including the ones where the stock blends into the blade shall be analysed according to Annex B.

# Equivalent diameter in way of notches

In way of key notches or square section, the equivalent diameter shall be defined as explained in Figure 6.

Key

1	 Initial diameter	2 :Square side	3 :Equivalent diameter

Figure 6 — Determination of equivalent diameter in way of key notches or square section

# Rudder bearings, pintles and gudgeons

## Bearing arrangement

Bearings may be plain bearing (cylindrical conical, spherical, etc), or roller bearings, or any arrangement of the two kinds.

If bearings are of roller type, they shall be designed to operate in sea water or be adequately protected by a device preventing them to operate in water (stuffing box, lip gasket, etc).

Unless specifically engineered, plain bearings shall have a length included between 1,2 and 1,5 d, d being the diameter of the rudder stock or pintle.

Plain bearing bushings shall be secured in their support against rotation and translation.

Bearings made with hygroscopic material expand with time when they absorb water. The shall be designed and engineered (specially for free plays) so that there is no risk of seizing or blockage due to material expansion and/or thermal expansion due to ambient temperature or friction.

As any material flexes under bending, bearings that can follow this bending (knee or knuckle bearings) will induce a better holding and a freer rotation. This is particularly true for material with low ratio E/d like titanium that bend significantly under load when designed according to equations of Article 8.

9.2	Bearing forces  (Reactions)

### Type I (spade)

Ru=FZBhu		is the load on upper (deck, etc) bearing										(N)    (19)

Where 	hu is the vertical distance between the centres upper and lower bearing (see Figure 1)

RL=Ru+F	is the load on lower (hull) bearing 											(N)    (20)

### Types II, III and IV (rigid skeg)

R=0,5 F	is the load on the upper bearing and lower bearing								(N)    (21)

NOTE	For simplification, the centre of effort is considered at mid-span

### Type V (rigid skeg)

RL=Ftha		is the load on the lower (skeg) bearing										(N)    (22)

Ru=RL+F	is the load on the upper (hull) bearing 										(N)    (23)

NOTE	It is the sum of the absolute values of RL and F as they may be in opposite directions

Where

- ha is the vertical distance between the centres of the upper and lower bearing

- t is the location of  rudder area centroid below the lower pintle (might need a minimum value here)

# Rudder stock structure and rudder construction

## Rudder stock structure

The rudder shall be designed to transmit the torque from the blade to the stock.

This is usually done by a set of arms, rods, connecting plates, frp lamination, etc. These shall be designed not to exceed d in any of their elements, under M and T.

If the connection between the rudder stock and its structure is made by welding, the mechanical properties shall be the as welded ones in the areas affected by welding heat.

## Rudder construction

The rudder construction and structure shall be able to withstand the hydrodynamic pressure loads and transfer them to the rudder stock or pintles.

If the rudder stock does not extend to the bottom of the rudder, the rudder shall have below the end of the rudder stock and in way of the rudder stock axis, at least the bending and twisting resistance defined below, according to the rudder arrangement.

## FRP rudder blades

### Blades with a core

#### Skins on a foam or equivalent properties core

If the rudder blade is made of a FRP skin on a core:

If this core is made of foam, this foam shall be made of PVC with a density of at least 64 kg/m3, or a foam at least an equivalent compressive and shear load (see sec 10.2 in 12215-5).  This foam shall be able to transfer the loads, usually by bearing load from the blade to the rudder stock or pintles structure, or be properly reinforced.

wr=1,1⋅k5⋅(115Lwl+150) is the minimum required fibre mass per area                        (g/m2)	(24)

Where

- wr  is the fibre mass per area of the rudder blade,										(g/m2)

- k5 	= fibre type factor, where:

  - k5 = 1,0 	for E-glass reinforcement containing up to 50% of chopped strand mat by weight

  - k5= 0,9 	for continuous glass reinforcement (i.e. bi-axials, woven roving, uni-directionals)

  - k5= 0,7 	for continuous reinforcement using aramid or carbon or hybrids thereof

Skins on a wood or plywood core

No more requirements than the general requirements of 11.1

### Blades skins without internal core

The blades shall be strong enough or properly stiffened, in order to transfer the loads to the rudder stock

It is considered reasonable that the skin fibre mass is 2 wr

## Non-FRP rudder blades

If any case, the pressure and shear loads on the skins shall be transferred to the rudder stock with an appropriate device, with normal and shear stresses not greater than the values required or implied by equation B1.

- (normative)

Metal for rudder stock

  - Metal rudder stock material

Metal rudder stocks may be made with one of the metals displayed in Table A1, with the following recommendations and restrictions.

    - General

Welding generally reduces corrosion resistance, mechanical properties and impairs heat treating. Therefore:

- in way of significant welding, one shall use the as welded properties given in Table A1 ;

- there shall be no "end to end" joint rudder stocks using welding within 0,15 hb from the point of maximal mechanical stress, unless specifically engineered.

    - Stainless steel.

AISI 316L, with low carbon content has better corrosion resistance than 316, but they are both recommended.

AISI 304 or 304L are not fully recommended for use under waterline as there is a risk of crevice corrosion.

On 17-4 PH steel, welds shall only be used as point or tack welding, to lock elements in place, not in elements needing any significant mechanical stress. As these steels have moderate corrosion resistance, d in Table 4 for 17-4 PH is lower than its y .

DX 45 (AISI F51) may be structurally welded, provided it is done in neutral atmosphere, preferably argon.

    - Aluminium alloys.

Due to their poor corrosion resistance in seawater Series 7000 alloys shall be avoided, unless in specific cases. In any case, information to the user on the risks and on the required time checking and maintenance, shall be included in the owner's manual

    - Titanium alloys

As Ta 6 V has high mechanical properties, it allows small diameter rudder stocks. As its elastic modulus is moderate (45 000 N/mm²), this might induce large deflection. One shall therefore check that the deflection of the stock under the nominal force F does not impair the functioning of the rudder stock not the resistance and watertightness of the rudder tube.

  - Design stress for metallic rudder stock

The values of Table A1 shall be used for the metals shown unless derived from tests.

If derived from tests the mechanical properties of u and y used in equation 12 shall be 90 % of the mean relevant tested value or the mean value minus two standard deviations whichever is the lesser.

Table A1 — Values of d for metallic rudder stock

Table to be completed.

- (informative)

Complex composite rudder stock design

  - Scope

Annex B is intended to apply to composite stock and blade arrangements where the laminate schedule is likely to involve complex unidirectional and bi-axial fibres with respect to the stock axis and these fibres may be tapered out as the stock passes down the rudder blade. The rudder blade is assumed to be effectively attached to the stock and may be considered to be capable of making a contribution to the strength and stiffness of the component.

  - Use of design loads

The applied bending moments and torques may be taken directly from Sections 6 and 7.

Bending moments and torques at other vertical positions may be may be made by considering only the blade area below the point under consideration.

Bending moments shall be determined by assuming that the side force given in equation (7) is uniformly distributed over the blade area in a vertical sense. In the case of an unsupported blade below the point under consideration, the vertical lever may be based on Table 3.

Torques levers shall be based on the mean chord of the area below the point under consideration and the levers given in Table 4.

  - Design Criteria

The strength of the stock and/or blade is to be assessed at a number of vertical locations

    - Strength criteria:

For FRP rudder stocks, equation (B1) shall be fulfilled:

σσu2+ττu2<0,25                                                                                  	(B1)

where

- σu  is the smaller of the ultimate tensile and compressive strength // to the stock axis   (N/mm²)

- τu   is the minimum shear strength // to the stock axis			    (N/mm²)

- See B.4

    - Stiffness criteria:

The flexural rigidity at the neck is not to be less than 50% of that of a AISI 316 stock:

EIneck≥0,5.208 000.π64d3164=5 105 d3164 												(Nmm²)    (B2)

Where d 316 is derived from equation (13).

  - Methods of analysis

The strength and stiffness of the stock and blade may be assessed using one of two methods:

- Only the UD fibres are considered as effective in carrying the bending moment, with only the bi-axial or diagonal fibres being considered effective in carrying the torque

- All fibres are effective

If the first method is used, the component will be considered satisfactory if the bending stress in the unidirectional fibres is less than 50% of the minimum of the ultimate compressive (or tensile) AND the shear stress in the other fibres is less than 50% of the ultimate shear strength.

In the second method, a laminate analysis using classical lamination theory shall be performed and the component under combined bending and torsion must satisfy equation B1.

  - Other considerations

    - Skin wrinkling

In the case of a blade face in compression, the compressive stress shall not exceed 50% of the face wrinkling strength.( see 12215-5 for explanations)

σc=0,6⋅3Es⋅Ec⋅Gc 	(N/mm²)		(B3)

where

- σc	is the design stress in compression  of the outer and inner skin 					(N/mm2)

- Es	is the compressive elastic modulus of skins, in 0°/ 90° in-plane axis of panel		(N/mm2)

- Ec	is the compressive elastic modulus of core, perpendicular to skins,				(N/mm2)

- Gc	is the core shear modulus, in the direction parallel to load,						(N/mm2)

    - Semi-spade rudders.

      - General

The horn provides an extra support to the rudderstock. As the rudderstock has three supports, it is a statically indeterminate structure. Compared to a spade rudder, the maximum bending moment on the rudderstock is dependent on the relative stiffness of the horn compared to the stiffness of the stock.

Annex A is intended to be used when it is required to determine the actual support provided by the skeg (full or partial).  Annex A need not be used if the skeg is treated as flexible and the stock scantling is obtained on the basis of treating the rudder as a spade.

      - Rudder with partial horn

The rudder area A is divided into 3 parts (See figure 6).

A=A1+A2+A3	is the sum of the three sub areas of the blade						    (m²)      (B4)

where Ai=(bti+bbi) hi are the values of sub areas with i =1,2,3

Mn=C1 (hb2−hn2) (0,5+0,125 u)+C2 hc2 (0,5+hbhc−0,25 u)1+u (1+hahb⋅ΙbΙa⋅EbEa) the bending moment at the neck (Nm)  (B5)

Mp=C2⋅hc22   is the bending moment on the rudderstock at the pintle						     (Nm)    (B6)

Tn=PHA (A1⋅r1+A2⋅r2+A3⋅r3)  is the Torque at the top of the rudderstock and neck	     (Nm)    (B7)

Tp=PHA (A2⋅r2+A3⋅r3) is the Torque at the pintle									     (Nm)    (B8)

where

C1=F (A1+0,5⋅A2)A (hr−hc) 	C2=F (A3+0,5⋅A2)A hc	andu=hb33 Ιb Eb hhlb2GhΙh+hh23 Ιh Eh are non dimensional coefficients

r1=0,2 bc1−x1  is the lever radius that shall not be taken less than 0,125 bc1  				(m)

r2=0,2 bc2−x2  but shall not be taken less than 0,125 bc2 								(m)

r3=0,33 bc3−x3  but shall not be taken less than 0,125 bc3  								(m)

bci are the horizontal lengths of the rudder sun areas at the centre of area Ai, and xi are the distance from the leading edges to the centreline of the pintle.

Ia and Ea are respectively the mean second moment (cm4) and Elastic modulus of the upper rudderstock.

Ib and Eb are respectively the mean second moment (cm4) and Elastic modulus of the rudder (blade +stock) between neck and pintle.

Ih and Eh are respectively the mean second moment (cm4) and Elastic modulus of the rudder horn.

Jh and Gh are respectively the mean polar moment (cm4) and Shear modulus of the rudder horn.

Jh=4 a ² ts is an approximation where :

a is the mean horizontal area enclosed by the outer perimeter of the horn plating 					(cm²)

t is the mean thickness of the horn plating 													(cm)

s is the mean median perimeter of the horn plating 											(cm)

Figure 7 schematizes the dimensions and the vertical variations of the bending and twisting moments.

From the values of  M and T, Section 7 shall be used to determine the rudderstock required dimensions.

Load on the upper bearing 	R1=Mnha											             (N)     (B9)

Load on the neck bearing 		R2=R1 (1+hahb)+C12 hb(hb−hn)²−Mphb				             (N)    (B10)

Load on the pintle bearing 		R3=F+R1 −R2									             (N)    (B11)

where F is the force on the rudder defined in equation (7).

Figure B 1 — Rudder with a partial horn

      - Rudder with full horn

This is a simplified case of the previous one, with A1 = A , A2 = A3 = 0, hc=0 and hb # hh # hr

Mn=Fhb (hb2−hn2) (0,5+0,125 u)1+u (1+hahb⋅ΙbΙa⋅EbEa) the bending moment at the neck 						    (Nm)   (B12)

Mp=0   is the bending moment on the rudderstock at the pintle (i.e. simply supported)

Tn=F r   is the Torque at the top of the rudderstock and neck								    (Nm)   (B13)

Tp=0 is the Torque at the pintle (i.e. simply supported)

where u=hb23 Ιb Eb lb2GhΙh+hb23 Ιh Eh

Load on the upper bearing 	R1=Mnha											             (N)    (B14)

Load on the neck bearing 		R2=R1 (1+hahb)+F2 hb2(hb−hn)²				                    (N)    (B15)

Load on the pintle bearing 		R3=F+R1 −R2									             (N)    (B16)

Figure B 2 — Rudder with a full horn

      - Horn load and scantlings

The load on the horn is the pintle bearing load R3 defined above in equation (B11) or (B16)

Note: The hydrodynamic load on the horn is generally considered in this part of ISO 12215, it may need consideration if the horn has and area greater than 25% of the one of the rudder.

Mh max= R3 hb 	is the maximum horn bending moment at the junction horn/hull 				(Nm)	 (B17)

Th max= R3 lb 		is the maximum horn twisting moment at the junction horn/hull 				(Nm)	 (B18)

Fh x= R3 		is the maximum horn shear load						l 				(N)		 (B19)

These moments vary linearly from the pintle to the junction horn/hull.

.

- (informative)

Technical background

  - Background documents

    - Sailing boats

As the ABS Guide for Offshore Racing Yachts is frequently used on sailing boats, it has been liberally used as a basis for this part of ISO 12215, but however with some changes.

    - Motor boats

For motor boats with slow or moderate speed the same analysis as for sailing boats was applied.

For motor boats with a maximum speed above 10 knots, the values of CR were based on an article from Donald Blount in the August 2002 issue of Professional Boatbuilder. This article give practical CR  values for rudders with a geometric aspect ratio of 1,5 at 20 and 40 knots.

  - Hypothesis of calculation

    - General

The allowable strength (0,9 y or 0,5 u) is in line with Parts 5 and 9 of 12215-5 but not exactly the same as ORY.

Also a design category factor fwR is introduced. It is the same as in part 9 and slightly different from part 5. This factor allows for a lower demand for categories C and D, where the craft is close to the shore and the sea conditions are less demanding.

    - Sailing boats

The force coefficient CR is not constant at 1,5 as in ABS/ORY, but variable with aspect ratio, as in GL rules.

The coefficient Kv is introduced to consider that light boats are capable of higher speeds, and it is based on the coefficient N of ABS/ORY. This coefficient is, of course, subject to adjustment after validation.

If multihulls may go much quicker than monohulls, they do not broach significantly, so their demand for high CR is less. This is why, at the time of publishing the draft, the requirements are the same for mono or multihulls. Using higher design speeds but lower CR would lead to equivalent F.

After some analysis, it became obvious that the GRP horn associated with metal rudderstock brought no support as their stiffness (EI product) is less than the one of the stock. Therefore the beneficial support from horn is only granted to "stiff" horns.

    - Motor boats

For motor boats with slow or moderate speed the sailing boat approach was followed. But as speed increased, the curves of CR  has been multiplied by a reduction factor to find the same value as Donald Blount's article for    =1,5. for the same speed.