Meeting Report

ISO Technical Committee 188, Work Group 5

Amsterdam, The Netherlands

November 18, 2008

Technical Committee 188 – Working Group 5 – Propulsion Systems

ISO/DIS 10088, Permanently installed fuel systems

- The ventilation requirement has been inadvertently left out of the fuel system standard.  This might be just an editorial change without having to go through another FDIS vote.

- Next action: The convener will verify and correct.

ISO/WD 25197, Electrical/electronic control system for steering, shift and throttle

- Revised a number of definitions, including those for joystick, maneuvering speed and cruising speed.

- The steering systems shall be tested according to IEC 60945. The U.S. recommended that ABYC S-31 be the testing criteria document. This option may be explored in the future.

- Added additional informative references:  IEC 60092-507 for 3-phase systems up to 50v DC.

- The failure alarm may have a mute function as long as the visual indication remains.

- Engines shall only start in the gear neutral position.

- Transition from maneuvering mode to cruising mode shall only be accomplished by input from the operator and shall not change modes without additional operator input.

- The vessel shall move in the same direction as the orientation of the control head relative to the craft.

- The WG needs to resolve questions of wireless control of the boat: 1. Not allowing wireless control while off the boat, or 2. Not allowing wireless control of another boat.

- The system needs a low battery indicator before control is lost and may need a minimum low battery power level before allowing wireless use.

- Next Action:  Continue review of the draft standard as a Committee Draft (CD).  The next meeting will probably be held in Guttenberg, Sweden in May 2009.