- Meeting Report

ISO Technical Committee 188 Work Group 22

London, England

June 4-6, 2006

- Technical Committee 188 – Work Group 22

This work group is responsible for developing the three parts of the stability and buoyancy standard, ISO 12217, for the Recreational Craft Directive.

Attending this meeting were delegates from the Sweden, UK, Norway, France, Canada, US, Japan, Norway and the Netherlands.

- A.  ISO 12217-1, Stability and Buoyancy, Assessment and categorization – Non-sailing boats with a hull length greater than or equal to 6m

- For catamarans, the assumed roll angle will be modified to include a length/beam ratio to accommodate the difference between narrow and beamy boats. This still needs to be validated.

- Should boats in category C and D not be required to be assessed if Alv < LhBh (6.4); or the proposed <.5LhBh?  This needs to be validated for small boats (US Marine action).

- Annex C, Methods for calculating downflooding angles, has been deleted.

- Annex D, Determining the curve of righting moments, will include the effects of free surface effects (after screening).  Look to require this test at the most dangerous condition, to be either full load/empty tanks or full load/max free surface (after screening).  Needs validation.

- Modified the Swamped buoyancy test mass for Design Category B (Table E.5).

- Next Action: Amended standard sent to ISO for a proposed DIS.

- B.  ISO 12217-2, Stability and Buoyancy, Assessment and categorization – Sailing boats with a hull length greater than 6m

- Quite a few comments received for revision of the STIX (Stability Index) assessment model.  The group determined that this tool works well and will revise this model judiciously so as to not have to revalidate the current fleet of acceptable sailboats over 6m.

- Work by the multi-hull sailing group must first be completed before a review of the standard is to start.

- Will add owners manual information concerning reefing wind speed.

- Next Action:  Continue review Part 2 at or around METS in November 2007.

- C.  ISO 12217-3, Stability and Buoyancy, Assessment and categorization – boats with a hull length less than 6m

- Amended the scope of this standard to continue to exclude inflatables except where ISO 6185 references this standard.

- The WG will petition Technical Committee 188 to assume development of stability standards for inflatables.

- Next Action: Send amended standard to ISO for DIS.

- The next meeting of the WG is tentatively scheduled to be held at METS in Amsterdam.

The information contained herein is prepared by NMMA staff and intended for informational purposes and informal background use only. It is not intended to serve as an official report of the meeting’s proceedings, actions or outcomes. This unofficial summary prepared by NMMA staff is not intended to generate any action on the part of any reader. The only recognized official transcript of any ISO meeting is the official minutes of the meeting generated by the meeting’s convener or sponsor.