Meeting Report

ABYC Engine and Powertrain PTC

Miami, Florida

October 8, 2008

Highlights of the ABYC Engine and Powertrain Project Technical Committee meeting:

- P-1, Installation of Exhaust Systems for Propulsion and Auxiliary Engines

- Several editorial items in P-1 have been corrected

- Segments of wet exhaust systems have been expanded to note:

  - if a water diverting or by-pass system is used to optimize system performance, and if a valve is used to regulate this, that the valve be only installed on the by-pass side of the system

  - In a dewatered exhaust system, with a drain below the water line, a seacock shall be installed as per ABYC H-27.  This seacock shall not be used to control flow to optimize the system.

- In a wet exhaust system, once the proper flow has been determined, the valve shall be locked out and labeled to indicate that the flow setting shall not be changed.

- Next Action:  Send to Tech Board for approval to publish.

- P-6, Propeller Shafting Systems

- Clearly defined shaft speed and shaft horsepower in the standard.

- Deleted the SAE exception that the keyway should follow SAE surface finishes.

- Removed all references to SAE J756, Marine propeller shaft couplings.

- Removed the recommendations for the ratios of propeller versus shaft diameter.

- Discussed whether Lloyd’s shaft sizing criteria can be used in shaft design.  More research needs to be done.

- The shaft design safety factor can be reduced if the shaft successful passes corrosion fatigue tests.  Determined that the minimum design coefficient shall not be less than 2.

- Removed the size requirement for the measurement between bearing struts.

- Removed all portions of the SAE J755 from the standard and added SAE J755 as a reference.

- Next action:  Send the standard out for consensus ballot.

- P-4, Marine Inboard Engines and Transmissions

- The location and arrangement of component parts of the engine shall provide access to those items necessary for manufacturer’s recommended periodic maintenance.

- The primary helm position shall have the full suite of instrumentation.

- Next action:  Send the standard out for consensus ballot.

- The PTC will request that the Technical Board move the generator set standard, A-27, to this PTC.

- P-4 and P-6 will be submitted to ANSI for approval.

- Next meeting of the PTC subcommittee at the call of the chair, maybe in Miami in February 2009.