Conference Call Report

ABYC Control Systems PTC

April 25, 2007

Highlights from the ABYC Control Systems Project Technical Committee Conference Call:

- An NMMA Action Item of a review of pneumatic system parameters has not yet been completed.

- P-27, Electric/Electronic Steering Control Systems

- Autopilots are not addressed in this standard.

- A handful of editorial revisions.

- Random vibration will be changed from 7.5gs (20 to 2000 Hertz) to an equivalent g^2/Hz number.

- Resonant vibration will be changed from 1gs and 4 gs peak (20 to 2000 Hertz) to an equivalent g^2/Hz number.

- Still need a vibration test for engine mounted components.  This will be submitted for the next review.

- Next action:  Update the random and resonant vibration conversions and send to Technical Board for approval.

- P-24, Electric/Electronic Propulsion Control Systems

- An electrical logic system shall require two operations to allow shifting from neutral.

- The system shall be operational after being stored in a temperature environment that ranges from -40F to 180F.  Time period of temperature pickling shall be addressed at the next revision (check UL test standards).

- Next action:  Send to Technical Board for approval.

- P-22, Steering Wheels

- Is the key and keyway intended as an assembly aid or is it also load bearing?  The PTC determined that the key and keyway is load bearing.

- Does the Figure 2 drawing of the key and keyway specify unique and allowable dimensions?  The PTC determined that these are not unique dimensions but representative of the system.

5.  The next meeting of the PTC will be held in June 2007.

The information contained herein is prepared by NMMA staff and intended for informational purposes and informal background use only.  It is not intended to serve as an official report of the meeting’s proceedings, actions or outcomes.  This unofficial summary prepared by NMMA staff is not intended to generate any action on the part of any reader.  The only recognized official transcript of any ABYC PTC meeting is the official minutes of the meeting generated by the meeting’s convener or sponsor.